Pre-Research Notes

AI for Junction Temperature Estimation in SiC/GaN Power Modules — EV Traction Inverters

IEEE VPPC 2026 · Special Session 6 · INSA Strasbourg

Section 1 — The EV Powertrain: Why the Inverter Exists

In any electric vehicle, the battery stores energy as DC — direct current at a fixed voltage, typically 400 V or 800 V. The electric motor, however, needs AC — alternating current with variable frequency to control speed and torque. The device that performs this conversion efficiently, at high power, thousands of times per second, is the traction inverter.

The traction inverter is a box of switches. It connects and disconnects the battery from the motor windings in precise patterns to synthesize a sinusoidal AC waveform. Those switches are power transistors. For decades, the transistor of choice was the Silicon IGBT (Insulated Gate Bipolar Transistor).

Why not just use a DC motor?

A DC motor requires a commutator and brushes to reverse current direction mechanically in the rotor windings. At EV power levels (50–300 kW) this creates critical problems:

- Brushes wear down, produce carbon dust, generate heat and electrical arcing — maintenance is constant.

- At high speed, the commutator can fly apart from centrifugal force.

- Arcing generates EMI that disrupts nearby electronics.

The AC motor (specifically the PMSM — Permanent Magnet Synchronous Motor used in most EVs) has none of these issues. The rotor is just steel and magnets — nothing touching it electrically. No wearing parts beyond the bearings.

The AC motor is mechanically simple, extremely robust, capable of very high-power density, highly efficient across a wide speed range, and capable of regenerative braking naturally. The inverter is the price you pay for using a vastly superior motor.

Section 2 — From Silicon to SiC/GaN: The Wide Bandgap Revolution

Silicon IGBTs work, but silicon as a material hits a fundamental ceiling. It cannot switch very fast, cannot manage very high voltages efficiently, and generates significant heat during switching. In an EV inverter switching at 10–20 kHz, those losses require a large, heavy cooling system.

What is a semiconductor, and what is the bandgap?

A semiconductor is a material with electrical conductivity between that of a conductor and an insulator. This property allows it to control the flow of electrical current. Semiconductors have a bandgap — the energy required to move an electron from the valence band (where electrons are bound) to the conduction band (where they move freely and carry current).

Band gap comparison.

- Silicon (Si): ~1.1 eV

- Silicon Carbide (SiC): ~3.26 eV

- Gallium Nitride (GaN): ~3.4 eV

SiC and GaN are called wide bandgap (WBG) semiconductors. The wider bandgap means the material can manage higher electric fields, higher temperatures, and switch faster before breaking down.

Key property comparison

- Small bandgap (Si): high — Large bandgap (SiC/GaN): lower but manageable: Conductivity

- Si: poor — SiC/GaN: excellent: Temperature stability

- Si: high — SiC/GaN: very low: Leakage current

- Si: limited — SiC/GaN: excellent: High voltage capability

- Si IGBT: 10–20 kHz — SiC/GaN MOSFET: 50–200 kHz: Switching speed.

What this means for the EV

In practical terms, the switch from silicon IGBTs to SiC MOSFETs in the traction inverter brings:

- Much lower switching losses and lower conduction losses

- Less energy wasted as heat → more battery-to-wheels efficiency → more range.

- Smaller, lighter cooling systems

→ Example: if 20 km of driving consumes 10% of battery energy with silicon, the same journey might use only 5% with SiC — doubling effective range for that segment.

GaN goes even further in switching speed but currently manages lower voltages (under 650 V), making it more suited to onboard chargers and DC-DC converters rather than the main traction inverter. SiC dominates the traction inverter space.

Section 3 — The Traction Inverter Circuit

The traction inverter uses six transistors arranged in three pairs — a three-phase bridge. Each pair (called a half-bridge leg) controls one phase of the motor.

The half-bridge principle

For any single motor phase, two switches are connected in series between the positive and negative DC rails. The motor winding connects at the midpoint:

- S1 closes (S2 open): current flows from + rail → through motor winding → to − rail.

- S2 closes (S1 open): current flows in the opposite direction through the winding.

- Alternating S1 and S2 rapidly produces AC at the winding. S1 and S2 are never closed simultaneously — that would short-circuit the battery.

The full three-phase bridge

Three half-bridge legs side by side give six switches total (S1–S6). The controller fires them in coordinated patterns — Pulse Width Modulation (PWM) — to produce three sinusoidal voltages 120° apart in phase. This is exactly what the PMSM needs to generate a rotating magnetic field and produce torque.

Each of these six switches is a SiC MOSFET. Every switching event — tens of thousands per second — deposits a pulse of heat directly into the semiconductor junction.

Section 4 — The Thermal Problem: Why SiC Creates a New Challenge

SiC is more efficient than silicon, but the thermal problem does not disappear — it transforms into something harder to manage. Three specific characteristics of SiC make classical thermal management methods insufficient:

- Higher operating temperatures: SiC is deliberately pushed closer to its 175°C junction temperature limit (vs 150°C for silicon IGBTs) to extract the efficiency gains. The safety margin shrinks dramatically.

- Much faster thermal dynamics: SiC switches 5–10× faster than silicon. Junction temperature spikes and recovers on microsecond timescales during each switching event. A case-mounted NTC thermistor has a thermal time constant of several seconds — it is completely blind to what happens at the junction during fast switching. At 10 kHz with silicon this was acceptable. At 50–100 kHz with SiC it is not.

- Different and less understood degradation mechanisms: Bond wire liftoff, solder fatigue, and die cracking in SiC modules behave differently under thermomechanical cycling than in silicon IGBTs. There is far less historical data. The thermal impedance models validated for silicon do not transfer directly to SiC.

→ The combination — higher temperatures + faster dynamics + less understood aging + less data — is exactly the environment where physics-based models struggle and where AI earns its place. Silicon was manageable with classical tools. SiC is not.

Section 5 — How AI Estimates Junction Temperature from Electrical Signals

The junction temperature Tj cannot be measured directly during operation — placing a sensor inside a running power module would destroy it. The key physical insight enabling an AI solution is that SiC MOSFETs have electrical properties that are sensitive to temperature.

The thermo-sensitive electrical parameter (TSEP)

The most important TSEP is Rds(on) — the on-state resistance of the MOSFET when it is fully conducting. When the junction is hotter, electrons move more slowly through the semiconductor lattice, and resistance increases in a measurable, predictable way. This means temperature information is already encoded in signals the controller is measuring continuously — no new hardware is required.

The challenge: Rds(on) depends not only on temperature, but also on gate voltage, drain current, and the aging state of the module. It is not a clean one-to-one mapping. That is exactly where AI is needed.

The four-stage estimation chain

Stage 1 — Physical signals (already available):  The inverter controller continuously measures gate voltage Vgs, drain current Id, drain voltage Vds, and case temperature Tc via a cheap NTC thermistor. No additional sensors required.

Stage 2 — Feature extraction (signal processing):  From raw signals, compute derived quantities: Rds(on) = Vds/Id during the on-state; slope of the switching transient (how fast voltage rises during turn-off); thermal ripple amplitude. This step is classical signal processing — not AI yet.

Stage 3 — AI model (the core contribution):  A trained model maps the extracted features to junction temperature. Architectures used include LSTM (Long Short-Term Memory networks, which capture temporal dependencies in the thermal state) and physics-informed neural networks (which embed known thermal equations as constraints, enabling generalization beyond training data). Ground truth Tj for training is obtained in the lab only — by pointing an infrared camera at the die surface during controlled experiments. Once trained, the model runs on the embedded controller with no camera needed.

Stage 4 — Feedback control:  The estimated T̂j feeds back into the PWM controller in real time. If the estimate approaches 175°C, the controller reduces switching frequency or current limit — protecting the device proactively rather than waiting for a catastrophic thermal cutout.

Why a simple formula is not enough.

A lookup table or fixed formula breaks down because Rds(on) depends on temperature, current, gate voltage, and aging state simultaneously — four interacting variables with nonlinear relationships. A classical RC thermal network (Foster or Cauer model) can model heat flow but cannot account for aging-induced changes in thermal impedance. An AI model trained across these conditions learns the full nonlinear mapping, and a physics-informed variant can extrapolate safely by respecting the known physics as hard constraints.

Section 6 — Research Gap and Paper Motivation

This pre-research establishes the motivation for the survey paper. The gap can be stated precisely:

No reliable, generalizable, real-time AI estimator for SiC junction temperature currently exists that works under real EV drive cycles and accounts for aging — and this is the frontier the paper will map.

The three open problems that the survey will cover:

- Junction temperature estimation: AI models mapping electrical TSEPs to Tj in real time, replacing slow thermal sensors.

- Aging and remaining useful life (RUL) prediction: detecting degradation state from measurable electrical signatures, enabling predictive maintenance.

- Thermally aware real-time control: closed-loop controllers that use the AI Tj estimate to continuously operate the device at its true thermal limit, maximizing performance while preserving lifetime.

→ Target venue: IEEE VPPC 2026 — Special Session 6: Data-Driven and AI-Based Methods for Advanced Electrified Transportation Systems. Co-chair: Tedjani Mesbahi, INSA Strasbourg. Submission deadline: 1 May 2026

The main databases to use.

IEEE Xplore (ieeexplore.ieee.org) — this is your primary source. VPPC, ECCE, APEC, PELS, and Transactions on Power Electronics are all IEEE publications and directly relevant. As an INSA student you have institutional access.

Google Scholar (scholar.google.com) — good for finding papers quickly and seeing how many times a paper has been cited. High citation count = foundational paper worth reading.

Science Direct / Elsevier (sciencedirect.com) — journals like Microelectronics Reliability and IEEE Transactions on Industrial Electronics are important for this topic.

ResearchGate (researchgate.net) — authors often upload their own papers here for free access even when the journal is paywalled.